ASSIGNMENT - 11

CN-LAB

Submitted To:

Sir Rasikh Ali

Submitted By:

Muhammad Talha (188)

Submitted On:

10/30/2024

Department of Software Engineering

Superior University, Lahore

TASK:

What is “DHCP, VLAN & DNS”, explain with Example (draw structure in cisco)

Solution:

1. DHCP (Dynamic Host Configuration Protocol)

DHCP automatically assigns IP addresses and other network configuration details (subnet mask, gateway, DNS server) to devices on a network, saving time compared to manual configuration.

Example:

- A PC connects to a network.

- The DHCP server assigns it an IP address (e.g., 192.168.1.10) and other settings like 255.255.255.0 (subnet mask) and 192.168.1.1 (default gateway).

- The PC can now communicate on the network.

2. VLAN (Virtual Local Area Network)

VLANs logically segment a physical network into multiple separate networks to improve performance and security. Devices in different VLANs cannot communicate unless explicitly allowed through routing.

Example:

- A company has one switch and three departments (Admin, Sales, IT).

- Create VLANs:

  - VLAN 10: Admin

  - VLAN 20: Sales

  - VLAN 30: IT

- Devices in VLAN 10 cannot access VLAN 20 or VLAN 30 directly, ensuring separation.

3. DNS (Domain Name System)

DNS translates human-readable domain names (e.g., www.google.com) into IP addresses (e.g., 142.250.190.14) so devices can locate and connect to servers.

Example:

- You type www.google.com in a browser.

- The DNS server resolves it to 142.250.190.14.

- Your device connects to Google’s server using this IP.

- DHCP (Dynamic Host Configuration Protocol)

enable

configure terminal

ip dhcp pool LAN

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

exit

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

- VLAN (Virtual Local Area Network)

enable

configure terminal

vlan 10

name Admin

exit

vlan 20

name Sales

exit

interface fa0/1

switchport mode access

switchport access vlan 10

exit

interface fa0/2

switchport mode access

switchport access vlan 10

exit

interface fa0/3

switchport mode access

switchport access vlan 20

exit

interface fa0/4

switchport mode access

switchport access vlan 20

exit

3. DNS (Domain Name System)